Crude Rapeseed Oil

Appearance light dust

Smell and taste characteristic to rape seed oil

Colored number mg, iodine, max 90

Acid value, mg KOH / g, max 6

Moisture and volatile matter, %, max 0.25

Un fat mixture, %, max 0.2

Flash point, C °, min 230

Unsupponification, %, max 1.5

Soap Number mg/KOH 165-200

Phosphorus matter recounting for stearooleoletsitin,%, max 2.0

Mass of eruk acid, %, max 3.0

Crude Degummed Rapeseed Oil

Density(15 °C) Kg/m³ 900-930 DIN EN ISO 12185

Flash Point P.-M °C Min. 220 DIN EN ISO 2719

Cinematic Viscosity(40 °C) mm²/s max. 36,0 DIN EN ISO 3104

Lower cloric value Kj/kg max. 36.000 DIN 51 900-2

Cetane Number ./. min. 39 IP 498

Carbon Residue Mass n.C. % (m/m) max. 0,40 DIN EN ISO 10370

Iodine Number g Jod/100g 95-125 DIN EN 14111

Sulphur Content mg/kg max. 10 DIN EN ISO 20884

Contamination mg/kg max. 24 DIN EN 12662

Erucic Acid Value mg KOH/g max. 2,0 DIN EN 14104

Oxidation Stability 110 °C h min. 6,0 DIN EN 14112

Phosphorus Content mg/kg max. 12 DIN EN 14107

Content of alkaline earth

Metals (Ca+Mg) mg/kg max. 20 E DIN EN 14538

Oxid Ash Content % (m/m) max. 0,01 DIN EN ISO 6245

Water Content K.-F. mg/kg max. 750 DIN EN ISO 12937

Crude Palm Oil

FFA (%) MAX 0.10

MOISTURE & IMP MAX 0.10

IV: 48- 56

MELTING POINT): 38

OXIDATIVE STABILITY: MIN.12

DENSITY, 20°C - KG/M3 – 912

DENSITY, 50°C-KG/M3-892

FLAM POINT °C -360

FLASH POINT °C -320

VISCOSITY, 50°C -MM2/S-28

CETANE NUMBER 50

CALORIC VALUE KJ/KG -MIN. 37000

SULPHUR MG/KG -3

PHOSPHOR MG/KG -1.6

CARBON RESIDE WT.%- 0.30

Crude Sunflower Oil

- FFA (%) NEN-EN-ISO 660 max 3.0

- Moisture (%) NEN-EN-ISO 662 max 0.5

- Impurities (%) NEN-EN-ISO 663 –

- Colour Lovibond (5¼ inch) NEN 6308 –

- Peroxide Value at loading (meq/kg) NEN-EN-ISO 3960 –

- Relative density at 20oC NEN 6311 –

- Iodine value (g 12/100g) NEN-EN-ISO 3961 –

- Flash point oC - Min 121 oC

Crude Soybean Oil

FFA (As oleic with a molecular weight of 282): Basis: 1.00%, Maximum: 1.25%

1.06% to 1.15% - discount to be 0.9% of contract price

1.16% to 1.25% - discount to be 1.2% of contract price

Up to 0.021% - discount to be 0.2% of contract price

Lecithin (expressed as Phosphorous): Basis: 0.020%, Maximum: 0.025%

Sediment (Gardner Break test): Maximum: 0.10%

Impurities (insoluble in petrol ether): Maximum: 0.10%

Moisture and Volatile Matter: Maximum: 0.20%

Unsaponifiable Matter (test as per N.S.P.A.): Maximum: 1.50%

Colour (Lovibond cell 1 inch): Basis: not darker than 50 yellow plus 5 red

Crude Degummed Soybean Oil

Free Fatty Acids (As OLEIC) Molecular Weight 282-Basis: BASE 1.00% MAX.1.25%

Moisture and Volatile Matter: 0.20% MAX.

Density (Specific Weight) at 25 C and 4 C Min. 0.9180 Max. 0.9225

Flash Point: MORE THAN 121 (250 F)

Saponification value Min. 188 Max. 198

Iodine value WIJS Min. 12 Max. 143

Impurities (Insoluble in Petrol Ether): 0.10% MAX.

Unsaponifiables Matter (test as per N.O.P.A.): Max. 1.50 pct.

Sediment (Gardner Break Test): 0.10% MAX.