Automatic Solar Cell IV EL Testing Machine

YH-PLIV

| Model | YH-PLIV |  |
| --- | --- | --- |
| Picture |  |  |
| Technical Parameter | Effective Test Range | 220 X 220mm |
|  | Test Range | 0W-20W |
|  | Lamp Spectrum | Lamp Spectrum: In line with IEC60904-9 and JISC8933 ( A Grade) |
|  | Light Intensity | 100mW/cm2 (continuous adjustable range 20-123W/cm2) |
|  | Light Intensity Non-Uniformity | ≤± 2% AAAA |
|  | Instability Degree of Irradiation | ≤± 2% AAA |
|  | Test Result Consistency | ≤± 0.5% AAA |
|  | Test Error of Electrical Properties | ≤1% AAA |
|  | Single Flash Time | 10-100ms (customized) |
|  | Measuring Voltage | 10-100ms (customized) |
|  | Measuring Current | 0 - 20A (resolution 1mA) |
|  | Test Parameters | Isc, Voc, Pmax, Vm, Im, FF, EFF, Temp, Rs, RSH |
|  | Power Demand | 220V/50HZ/2KVA |
|  | Production Capacity | The actual reference operating speed is 1500 pcs/ hour, and the average speed is about 2.4 seconds / tablet. (* it does not include the time of pick and place the cell, the speed is affected by the different test items and test accuracy) |
|  | Fragment Rate | ≤ 0.05% (this parameter mainly refers to the damage caused by the machine itself, excluding the damage of raw materials) |
|  | Defective Over Judgment Rate | about 6% A Grade cell is misjudged as B Grade/ Tested Qty |
|  | Missed Rate | ≤0.3% Calculation formula: Qty of missed Grade B / Tested Qty |
| Features | - Solar cell tester is a high-reliability, high-precision solar battery test equipment. - The equipment adopts high-power, long-life imported pulses xenon lamp as the simulator light source; - Imported ultra-high-precision four-channel synchronous data acquisition card to test data acquisition; - Professional superlinear electronic load ensure the test results are accurate; - Non contact detection to avoid secondary damage of battery; - It can be connected with the automatic line for online detection and automatic picking for detection and automatic grading; - The software can automatically identify defects without manual operation. |  |
| Environmental Requirements | - Solar panel tester venue requires greater than 3m×5m; higher than 2.5m special testing room; room illumination is less than 100 lux; - Room should be installed heating and air conditioning, indoor temperatures stable at around 25 ℃; - Power with 220v/2kw,and convenient connection, pay special attention to ensure that the equipment can reliable grounding grounding device; - Must have sufficient area under test battery connected to the test chamber temporary in order to ensure that the battery test to be tested with essentially the same test at room temperature; - The test room shall be equipped with a compressed air source interface, the air pressure of the air source to be stable at 5 - 8 Kg / C ㎡. |  |